JALCO AUSTRALIA PTY. LTD

Unit 1/8 Johnston Crescent, Horsley Park,NSW,2175

# POLLUTION INCIDENT RESPONSE MANAGEMENT PLAN

| Pollution Incident Response Management Plan Licence number: 21740 |  |
| --- | --- |
| Approved By: Rojli Rajon Position/Title: Site Operations Manager | Signature: Date: 20/11/2023 |
| Purpose: JALCO AUSTRALIA PTY. LTD holds an Environment Protection Licence with the NSW Environment Protection Authority (EPA) for 8 JOHNSTON CRESCENT HORSLEY PARK NSW 2175, As per the Protection of the Environment Operations Act 1997 (the POEO Act), the holder of an Environment Protection Licence must prepare, keep, test and implement a pollution incident response management plan (PIRMP) that complies with Part 5.7A of the POEO Act in relation to the activity to which the licence relates. If a pollution incident occurs in the course of an activity so that material harm to the environment (within the meaning of section 147 of the POEO Act) is caused or threatened, Jalco Australia Pty Ltd must immediately implement this plan in relation to the activity required by Part 5.7A of the POEO Act. A copy of this plan is kept at the main office building in reception on the premises and will be made available on request by an authorised EPA officer and to any person who is responsible for implementing this plan. This plan is also available on Jalco Website (https://www.jalco.com.au/contact-us/). Note: This plan has been developed in accordance with the Protection of the Environment Operations Act 1997, the Protection of the Environment Operations (General) Regulation 2022 and in reference to EPA’s Guideline: Pollution incident response management plans. |  |

| Environment Protection Licence (EPL) Details |  |
| --- | --- |
| Name of licensee: (including ABN) | JALCO AUSTRALIA PTY. LTD 45 075 091 833 |
| EPL number: | 21740 |
| Premises name and address: | UNIT 1/8 JOHNSTON CRESCENT, HORSLEY PARK, NSW, 2175 |
| Company or business contact details | Name: Michael Beaton Position or title: General Manager Manufacturing Operations Contract Manufacturing Business hours contact number/s: 02 8784 4950 After hours contact number/s: 0418 456 418 Email: Michael.Beaton@pactgroup.com |
|  | Name: Christine Kazzi Position or title: Health Safety Environment and Sustainability Manager Business hours contact number/s: 02 9757 6307 After hours contact number/s: 0410 220 892 Email: Christine.Kazzi@pactgroup.com |
|  | Emergency Contact: 1800 242 176 |
| Website address: | https://www.jalco.com.au (Jalco Group - AustraliaJalco ¦ Group) |
| Scheduled activity/activities on EPL: | Chemical Production |
| Fee-based activity/activities on EPL: | Soaps and Detergents Production > 5000T Annual Production Capacity |
| Pollution Incident – Person/s Responsible |  |
| Pollution Incident Response Management Plan Activation | Name of person responsible: Rojli Rajon Position or title: Site Operations Manager Business hours contact number/s: 0431 777 657 After hours contact number/s: 0431 777 657 Email: RojliM.Rajon@pactgroup.com |
|  | Name: Christine Kazzi Position or title: Health Safety Environment and Sustainability Manager Business hours contact number/s: 02 9757 6307 After hours contact number/s: 0410 220 892 Email: Christine.Kazzi@pactgroup.com |

| Pollution Incident – Person/s Responsible, Continued |  |  |  |
| --- | --- | --- | --- |
| Notifying relevant authorities | Name: Michael Beaton Position or title: General Manager Manufacturing Operations Contract Manufacturing Business hours contact number/s: 02 8784 4950 After hours contact number/s: 0418 456 418 Email: Michael.Beaton@pactgroup.com The General Manager Manufacturing Operations can nominate another General Manager or Site Representative to notify relevant authorities. |  |  |
| Managing response to pollution incident/ Control Coordinators | Name of person responsible: Rojli Rajon Position or title: Site Operations Manager Business hours contact number/s: 0431 777 657 After hours contact number/s: 0431 777 657 Email: RojliM.Rajon@pactgroup.com |  |  |
|  | Name: Christine Kazzi Position or title: Health Safety Environment and Sustainability Manager Business hours contact number/s: 02 9757 6307 After hours contact number/s: 0410 220 892 Email: Christine.Kazzi@pactgroup.com |  |  |
|  | Name of person responsible: Gopi Dhanekula Position or title: Engineering Manager Business hours contact number/s: 0448 311 389 After hours contact number/s: 0448 311 389 Email: Gopi.Dhanekula@pactgroup.com |  |  |
|  | Name of person responsible: Pat Omprakash Position or title: Manufacturing Manager Business hours contact number/s: 0425 305 863 After hours contact number/s: 0425 305 863 Email: Pat.Omprakash@pactgroup.com |  |  |
|  | Name of person responsible: Aj Afualo Position or title: Warehouse Manager Business hours contact number/s: 0419 166 115 After hours contact number/s: 0419 166 115 Email: Aj.Afualo@pactgroup.com |  |  |
|  | Name of person responsible: Shalini Singh Position or title: QA Manager Business hours contact number/s: 02 9757 6315 After hours contact number/s: 0404 818 746 Email: ShaliniS.Singh@pactgroup.com |  |  |
| Notification of Relevant Authorities |  |  |  |
| Fire and Rescue NSW / Rural Fire Service |  | Contact number/s: | 000 |
| EPA |  | Contact number/s: | 131 555 |
| NSW Health |  | Relevant Area Health Code: Contact number/s: | Liverpool Public Health Unit Business Hours 02 9794 0855 After Hours 02 8738 3000 |
| SafeWork NSW |  | Contact number/s: | 13 10 50 |
| Fairfield City Council |  | Contact number/s: | 02 9725 0222 |
| Notification of Relevant Authorities, Continued |  |  |  |
| Fairfield City Council |  | Contact number/s: | 02 9725 0222 mail@fairfieldcity.nsw.gov.au |
| Sydney Water St Mary |  | Contact number/s: | Ph: 02 4736 9153 Fax: 02 4736 9153 Emergency 24 hours: 13 20 90 |
| Roads and Maritime Services |  | Contact number/s: | 1800 814 813 |
| Notification of Neighbours and the Local Community |  |  |  |
| UPS Health Care Unit 2/8 Johnston Crescent, Horsley Park, NSW, 2175 02 9582 7232 02 8865 6903 Holman Industries Unit 1/10 Johnston Crescent, Horsley Park, NSW, 2175 02 9933 6500 (Press 4 General Enquires) Tennant Australia Unit 2/10 Johnston Crescent, Horsley Park, NSW, 2175 1800 226 843 (Press 6 General Enquires) Rhomberg Sersa Rail Group (RKR) Unit 3/10 Johnston Crescent, Horsley Park, NSW, 2175 02 9627 8200 Fienza Unit 2/12 Johnston Crescent, Horsley Park, NSW, 2175 07 3490 6700 (Head Office , Press 1) Speedpanel Unit 1/12 Johnston Crescent, Horsley Park, NSW, 2175 03 9115 6666 (Press 6 General Enquires) Lot 202 Under Construction (ESR) Johnston Crescent, Horsley Park, NSW, 2175 02 9506 1467 0403 454 674 Nu-Pure Beverages 4 Johnston Crescent, Horsley Park, NSW, 2175 1300 199 107 Williams Sonoma 2A Johnston Crescent, Horsley Park, NSW, 2175 02 8046 3400 DHL 2A Johnston Crescent, Horsley Park, NSW, 2175 02 9851 3736 ( Press 3 other safety emergencies) 02 9851 3734 Goliath Games (Crown and Andrews Pty Ltd) Unit 2/2 Johnston Crescent, Horsley Park, NSW, 2175 02 9898 3555 Hino Unit 1/2 Johnston Crescent, Horsley Park, NSW, 2175 02 9914 6629 02 9914 6631 Residences on Greenway Place, Horsley Park, NSW,2175 |  |  |  |
| In the event of a determined material harm incident, community notification will be undertaken by the Pollution Incident and Control Coordinators. When contacting adjacent companies and neighbours the following notification process is to be used: - Warnings: In the event of an incident, same day face to face contact and telephone notification will be employed to update affected landholders. - Updates: Follow-up telephone calls will be made to all landholders who were notified in the initial warning. Updated information will be provided if and or when it becomes available and necessary to be passed on. Updates will be provided to the community as follows: - Face to face contact or telephone call. - Letterbox drops. - Publication of updates on Jalco’s Website. - Emailing of updates; and - Doorknocking. |  |  |  |
| Description and Likelihood of Hazards |  |  |  |
| Pollutants that are applicable to Jalco Australia Pty Ltd Horsley Park include potential emissions to water, air, noise and land. The premises has identified the following hazards that can present harm to health or environment: - Spills and unintended discharge from storage of chemicals including: material transfer points ( unloading stations), DAF plant wastewater, chemical storage areas, above ground holding tanks and pipes and the generation or storage areas for waste materials; - Failure of plant and equipment including: ruptured containment tanks, uncontrolled release of gas, breakage of pipes, malfunctioning of valves, disruption to manufacturing processes, power failure and fire; - Failure to adhere with day, evening and night noise limits as indicated in EPL 21740; and - Offensive fugitive odour emitted from the site DAF plant, failure of the carbon filtration/ extraction system, fragrance containing products. The following risk matrix is based on PACT Group WHSE categorisation of the risk level on the likelihood and consequence of an event. |  |  |  |
| Risk Analysis – Likelihood vs Consequence (PACT Group WHSE Risk Matrix) Ratings:. Very High (A):; Immediate action by senior management required. High (B):; Senior Management attention required. Moderate (C):; Management responsibility must be specified. Low (D):; Manage by routine controls and procedures. Risk Analysis – Action Required |
| Pre-emptive Actions to be Taken |  |  |  |
| - Storage of Chemicals Area/ Operation; Hazard Cause; Hazard Consequence; Likelihood; Consequence; Risk Level; Pre-emptive Actions Required or In Place. Raw Material Store (Class 5.1,6.1, 8 & 9); Dislodgement from racking; Potential environmental release; Unlikely (2); Low (2); Moderate (C); The area is bunded in accordance with AS3780-2008 Incompatible materials are segregated Segregation for incompatible chemicals displayed Training provided for staff Racking designed to withstand standard workload, signages displayed on racks There is only one forklift at a time permitted inside raw material store that stages material for production. This eliminates any incompatible materials stored. Authorised and trained forklift drivers permitted in this area. Spillages are captured in blind sump drain Product decanted into another compatible or clean container. PPE must be worn in accordance with chemical SDS minimum requirement for minor spills safety boots, chemical resistant overalls, P3 face respirator and chemical resistant gloves. Minor spillages HAZCHEM spill kits are available. The spillage will be collected and placed in disposal bags. Disposable bag will be collected by certified waste collector. The liquid will also be transferred to DAF plant for treatment. The minimum PPE requirement for mix of incompatible goods or catastrophic nature wear self-contained breathing apparatus and chemical splash suit. Structural firefighter’s uniform will provide limited protection. Class 8 bases and acids are separated by bunding as per requirements. Dangerous Goods are kept at a designated storage location. Incoming receiver checks delivery docket and identify which storage area the goods will be stored and ensure immediate storage in correct area. Procedures for the retrieval and decanting of chemicals are in place. Spill kits are located nearby chemical storage areas. In case of spill, refer to SDS for appropriate handling.. Forklift misalignment, resulting in puncture of package; Mix of incompatible goods (exothermic reaction). Package dropped from forklift Toxic Effects of Chemicals to Human Health. Flammable Liquids Store (Class 3); Dislodgement from racking; Potential environmental release; Rare (2); High (4); Moderate (C); The area is bunded in accordance with bunding compliant with AS3780 -2008 Forklift drivers are competent drivers and undergo periodic training There is site stormwater containment Sprinkler protection Electrical equipment complying with AS/NZS 60079.14:2017 Training for forklift drivers in safe operation There is to be no smoking in the area. Ignition source control placard, comply with AS 1940-2017 Fire equipment such as fire reel hose, extinguishers, and hydrants Minor spillages HAZCHEM spill kits are available. The spillage will be collected and placed in disposal bags. Disposable bag will be collected by certified waste collector. Small spills can evaporate as it be captured in controlled extraction device. The chemical will safely be collected and pumped into an IBC to be treated if diluted or certified waste provider to appropriately discard. PPE must be worn in accordance with chemical SDS minimum requirement for minor spills safety boots, chemical resistant overalls, P3 face respirator and chemical resistant gloves The minimum PPE requirement requires Level A protection if risk of vapour or products of combustion that includes self-contained breathing apparatus, chemical protective suit designed with full encapsulation, inner and outer safety gloves that are chemical resistant. Steel capped chemical resistant boots. Class 3 Dangerous Goods are kept at a designated storage location. Incoming receiver checks delivery docket and identify which storage area the goods will be stored and ensure immediate storage in correct area. Procedures for the retrieval and decanting of chemicals are in place. Spill kits are located nearby chemical storage areas. In case of spill, refer to SDS for appropriate handling.. Forklift misalignment, resulting in puncture of package; Pool of flammable liquid, immediate ignition, and fire. Package dropped from forklift; Delayed ignition of flammable liquid and flash fire or explosion. Dropped flammable IBC during transportation Flammability of Chemicals Toxic Effects of Chemicals to Human Health. Bulk Acids and Bases Tanks (Class 8); Tank leak (leaks from valves, fittings, or pipework); Environmental release; Unlikely (2); Moderate (3); Moderate (C); Site stormwater containment The area is bunded in accordance with bunding compliant with AS3780 -2008 Segregation compounds of acids and bases High level sensors and alarms to control overflow of tanks Forklift drivers are competent drivers and undergo periodic training Minor spillages HAZCHEM spill kits are available. The spillage will be collected and placed in disposal bags. Disposable bag will be collected by certified waste collector. Preventative maintenance, routine inspections including annual inspection by qualified contractor. This include plan of management for pipe bridge. PPE must be worn in accordance with chemical SDS minimum requirement for minor spills safety boots, chemical resistant overalls, P3 face respirator and chemical resistant gloves. The minimum PPE requirement for mix of incompatible goods or catastrophic nature wear self-contained breathing apparatus and chemical splash suit. Structural firefighter’s uniform will provide limited protection. Dangerous Goods are kept at a designated storage location. Incoming receiver checks delivery docket and identify which storage area the goods will be stored and ensure immediate storage in correct area. Procedures for the retrieval and decanting of chemicals are in place. Spill kits are located nearby chemical storage areas. In case of spill, refer to SDS for appropriate handling.. Overfilling of tank. Potential impact from forklift, resulting in release; Mixing of incompatible goods (exothermic reaction) Toxic Effects of Chemicals to Human Health. Operator error (mixing of incompatible goods). DAF; Punctured or deteriorated IBC; Environmental Release; Unlikely (2); Low (2); Moderate (C); Natural ventilation Vents connected to balance, intermediate and sludge tank. The area is bunded in accordance with bunding compliant with AS3780 -2008 There is secondary containment for dosing equipment that will be placed on individual chemical bund including drip trays. For major spillage this will be captured in pits within DAF plant and recirculated to be treated. Routine and preventative maintenance carried out. Periodic forklift training provided to operators DAF plant will be roofed that will prevent flooding during heavy rainfall. PPE must be worn in accordance with chemical SDS minimum requirement for minor spills safety boots, chemical resistant overalls, P3 face respirator and chemical resistant gloves. Dangerous Goods are kept at a designated storage location. Incoming receiver checks delivery docket and identify which storage area the goods will be stored and ensure immediate storage in correct area. Class 5.1 hydrogen peroxide is kept in a cabinet suited for Class 5.1 storage. Procedures for the retrieval and decanting of chemicals are in place. Spill kits are located nearby chemical storage areas. In case of spill, refer to SDS for appropriate handling.. Leak from dosing equipment Toxic Effects of Chemicals to Human Health; Mixing of incompatible substances (exothermic reaction). Environmental Release. Toxic Smoke Emissions. Unloading Station; Leaks from valves, fittings, or pipework; Potential environmental release Mix of incompatible goods (exothermic reaction); Unlikely (2); Moderate (3); Moderate (C); Driver over bund in accordance with bunding compliant with AS2890 Segregation compounds of acids and bases. Signages of chemical and pumps allocated. Tanker driver to provide truck mechanics and hose conditions prior to attendance. Truck and hose are verified by Jalco warehouse receiver prior to discharge. Interlock system to close drain prior to commencement of pump out. Drain safe system compliant to WHS Regulation NSW 2017, Cl 357. Emergency stop available at each station and inside warehouse for failures or catastrophic encounters. Driver remains seated in unloading area monitoring discharge. Housekeeping inspection of area to ensure no spillages prior to tanker departure. Minor spillages HAZCHEM spill kits are available. The spillage will be collected and placed in disposal bags. Disposable bag will be collected by certified waste collector. Preventative maintenance, routine inspections including annual inspection by qualified contractor. This include plan of management for pipe bridge. PPE must be worn in accordance with chemical SDS minimum requirement for minor spills safety boots, chemical resistant overalls, P3 face respirator and chemical resistant gloves.. - Storage of Waste Area/ Operation; Hazard Cause; Hazard Consequence; Likelihood; Consequence; Risk Level; Pre-emptive Actions Required or In Place. Solid Waste manufacturing, production, warehouse and office; Waste not separated and categorised appropriately to allow for correct disposal Failure to collect waste based on agreed frequency with licensed waste provider; Incorrect disposal of waste Congested work and storage areas Illegal dumping of waste material; Rare (1); Very Low (1); Low (D); Wastes are separated into streams recyclable, returnable, metal, reuse, landfill. The site will obtain a compactor for cardboard materials that cannot be reused internally or returned to supplier. For plastic recycling there will be bottle and soft plastic bailers that are sent for recycling. Furthermore, site to determine whether bottler shredder onsite is feasible and environmentally friendly from energy perspective. Office and Lunch areas will have general waste, Comingle and recycling bins. Timber waste from pallets are reused or returned to supplier. Metal waste and will be recycled. Glass and biological waste is minimal at site and disposed of appropriately. Waste from manufacturing, production and office areas are collected daily as per agreement with licensed waste collectors. A destruction certificate is issued to site once legally disposed and attached to invoice. A waste management plan will be developed that captures various waste streams and waste minimisation (JHAP-WMP-002 Waste Management Plan).. Wastewater and potential discharge to stormwater; Failure to collect sludge from DAF based on agreed frequency with licensed waste provider Sludge hose removed outside bunded area and remanent leaks into drain directed to OCD tank due to damaged or unclean equipment Too much surfactant, or any other substances that disrupt chemical and biological balance in DAF plant Incorrect flocculant preparation (cationic or anionic) used Excessive rain causing overflow and lead to stormwater contamination as no physical separation between stormwater and wastewater system in production area; Disposal of wastewater Illegal dumping from waste provider Environmental release to stormwater Release of Chlorine and toxic gas Potential breach to Sydney Water Agreement; Possible (3); Moderate (C); High (B); A waste management plan will be developed that captures various waste streams and waste minimisation (JHAP-WMP-002 Waste Management Plan). Training provided to site on managing DAF plant. Maintenance that includes monthly inspection service, calibration undertaken by Integra. Sludge is collected by a licensed waste contractor – Envirowaste services. Enviro waste depot for wastewater is based in Yennora and is licensed to treated sludge. Transport and destruction certificate from Envirowaste is provided to site with invoice. Envirowaste contractor is trained to site processes and protocols that includes certification that hose used to collect sludge is of good condition. Furthermore, facility inside DAF bund is used to camlock hose to pipework and upon completion tap is used to wash remnants prior to returning hose in tanker. DAF operator monitors the sludge holding tank capacity daily. There is a high level alarm that triggers back to control panel. Tank capacity is available on control panel. Site to determine whether anionic or cationic flocculant is required depending on products manufactured. As per Sydney water agreement to discharge a roof was installed to protect sewer from an influx of storm water during a heavy rain event. Ventilation is adequate and vents are available on balance and sludge tanks that will eliminate toxic gases. Spill kit is located in DAF area, drain covers are available and located on hardstand. High level alarm sensor is integrated to batch control system to notify manufacturing and maintenance of a fault. The fault are interlocked to internal trade waste pumps. In the event of an overflow from wastewater plant, the wastewater is transferred to overflow tank and balance tank. If overflow exceeds the additional capacity will be pumped into temporary IBCs or pumped into enviro waste tanker. This prevents any possible overflow of wastewater entering other areas of site and offsite environmental release. DAF system is automated with limited manual handling required that reduces chemical spills. Wastewater from manufacturing and production is diverted to DAF plant for treatment. A requirement from Sydney Water Consent to Discharge is that treated wastewater must meet parameters before released into Sydney water sewer network. Composite and Discrete samples are collected at prescribed frequencies by Jalco/ Integra and tested by NATA certified third party laboratory. The results are forwarded to Sydney Water within 22 day timeframe. Sludge and other liquid waste such as rejected bulk (work in progress) are collected by enviro waste. The waste is collected upon site request. Functional description of DAF plant prepared by Integra is available. In addition, site to establish wastewater standard operating procedure.. Liquid Waste, Hazardous Waste and Special Waste; Failure to collect liquid waste based on agreed frequency with licensed waste contractor; Liquid waste spill or contamination from hazardous waste; Possible (3); Low (2); Moderate (C); A waste management plan will be developed that captures various waste streams and waste minimisation (JHAP-WMP-002 Waste Management Plan). Waste is separated on site, with liquid wastes removed by licensed waste contractor. Transport and destruction certificate from is provided by licensed contractor to site with invoice. Liquid and hazardous wastes are stored in bunded containers or IBCs. There is a blind sump located in raw material store that is pumped into IBCs should a major spillage occur. Spillages located inside the factory are rediverted to DAF plant for treatment. Spill kits are available on hardstand areas external and inside manufacturing, production and warehouse areas.. - Potential failure of Chemical/ Contaminated Water Storage Tanks Area/ Operation; Hazard Cause; Hazard Consequence; Likelihood; Consequence; Risk Level; Pre-emptive Actions Required or In Place. Potential failure of storage or mixing tanks; Tank leak (leaks from valves, fittings, or pipework) Uncontrolled spill from rupture Overfilling of tank; Toxic effects of chemicals to human health Release of chemicals to DAF plant Mixing of incompatible goods (exothermic reaction); Unlikely (2); Low (2); Moderate (C); Batch control system, protocols and procedures Storage holding tank are bunded in accordance to AS3780 -2008 as per chemical compatibility and are identified per zone. Each zone has an emergency stop that automatically shutdowns and ceases pumps, valves and tanks. Storage and mixing tanks are located inside the facility (Eastern side of building). Preventative maintenance, routine inspections including annual inspection by qualified contractor this includes pipe bridge. A plan of management for pipe bridge was developed and available for site. Spillages are captured inside bund and diverted to DAF plant for treatment. Where there is catastrophic failure and overflow of bund chemicals will be pumped into IBCs. IBCs will be collected by licensed contractor for disposal. Spill kits are available in the area for chemicals spilt outside bund. Training provided to staff on chemical incompatibility and handling annually.. - Potential Failure of Containment Retention Water Area/ Operation; Hazard Cause; Hazard Consequence; Likelihood; Consequence; Risk Level; Pre-emptive Actions Required or In Place. Potential failure of containment retention water including penstock isolation valve; Failure of penstock isolation valve Pit 111/1 Failure to test quality of water to be discharged Torrential Rainfall; Release of potentially harmful environment Substances to stormwater; Unlikely (2); Low (2); Moderate (C); Penstock isolation valve Pit 111/1 is located in UPS Carpark (Entry/Exit). The system has both manual and automated functions. Training provided to site by builder (Hansen & Yuncken) on process. The automated process will link to fire panel and can be triggered by the system. Preventative maintenance of penstock isolation valve including testing of manual and automated operation. The contaminated water release will be stored in 2350 tonne underground OSD tank. A licensed waste provider will be arranged to pump material. Envirowaste services are on a 24 hour call out, as per the procedures detailed in stormwater management plan. Shift supervisor/ manager must call Envirowaste services and escalate to maintenance, environmental health safety and operations managers. Discharge of treated wastewater to sewer is completed by a trained Jalco operator and only required when wastewater parameters are met. Site procedure will be in place for stormwater systems.. - Potential Failure to meet Noise Limits L2.1 Noise generated at the premises must not exceed the noise limits at the times and locations in the table below: For the purposes of demonstrating compliance with condition L2.1: a) Day means the period from 7am to 6pm Monday to Saturday and the period from 8am to 6pm Sunday and public holidays. b) Evening means the period from 6pm to 10pm. c) Night means the period from 10pm to 7am Monday to Saturday and the period from 10pm to 8am Sunday and public holidays. Area/ Operation; Hazard Cause; Hazard Consequence; Likelihood; Consequence; Risk Level; Pre-emptive Actions Required or In Place. Failure to meet noise limits; Diaphragm pumps Blow down of boiler Forklift traffic and constant beeping Reverse alarms on trucks; Noise exceeding set out in site EPL Community Complaints; Unlikely (2); Low (2); Moderate (C); 18mm marine plywood internal lining acoustic wall fixed inside purlins on southern elevation of Production area. Plywood lining has a minimum surface density of 10kg/m² and form continuous layer to full height 0.48mm steel. Four sided enclosure rooftops fans, minimum enclosure height 1.0m above fan height. Maintenance of relevant noise control equipment. During night period limit forklift movement on hardstand, include use of non-tonal reverse alarms for vehicles and forklifts. Electric forklifts are used for all internal and external operations to reduce noise. Waste collection permitted from 7am -10pm Monday – Friday as per SSDA. Blowdown of plant and equipment to occur once in 12-month period. Ventilation systems Boiler blowdown to occur once every 5 months during weekdays. Noise assessment completed within 12-month period and submitted to EPA within 30 days.. - Potential Release of Air Pollutants Within six months of commissioning, the Licensee must engage a suitably qualified air quality consultant to undertake a site-specific odour audit. The odour audit must; a) identify all significant sources of odour at the Jalco Manufacturing Facility premises, including, but not limited to; • The mixing tanks and bulk tanks carbon filter outlet • The filling lines and dispensary carbon filter outlet • The Wastewater Treatment Plant outlets (including the DAF, sludge storage tank and balance tank) Area/ Operation; Hazard Cause; Hazard Consequence; Likelihood; Consequence; Risk Level; Pre-emptive Actions Required or In Place. Potential release of air pollutants Emission Potentially Offensive Odour; Failure of control technology; Toxic effects of air pollutants to Human Health Release of potentially harmful environmental substances in air Dust; Unlikely (2); Low (2); Moderate (C); Carbon breakthrough monitoring and management by OEM that is serviced on a contractor basis. Odour assessment completed within 6 months of commissioning. Equipment is maintained in good condition and serviced as per manufacturers specifications. Housekeeping inspection conducted monthly include odour. Fragrance materials tightly closed and stored in allocated storage location. Tank vats remain closed when not in use. Damaged/ leaking containers removed or disposed of correctly. Residual product on drums/equipment cleaned after use and cleaning material disposed appropriately. Bunded areas cleared of waste liquid. Product positioned on spill tray bunds must be cleaned immediately. Clean up of spillages and identify/rectify leaks that contribute to fugitive emissions. Vehicles emissions minimised as no idling. Driver code of conduct instructs drivers to switch off engines when waiting to dock. Drivers switch off engines during unloading/loading process. Procedures developed to contain spillages and odour management. Training provided to employees. Service and Inspections for all equipment including: Boiler Carbon Filtration Systems. - Other Hazards: Light Pollution Area/ Operation; Hazard Cause; Hazard Consequence; Likelihood; Consequence; Risk Level; Pre-emptive Actions Required or In Place. Poor or inadequate lighting; Lighting design allowance for artificial light to shine outward and upward into the sky; Nuisance for neighbours Complaints; Unlikely (2); Low (2); Moderate (C); Solar panels used to reduce energy consumption. Minimal intensity lights are used and point in specific direction All lights at site are sensor or timed internally and externally. Glass windows allows for natural light to seep through.. |

Inventory of Pollutants

Jalco Australia Pty Ltd Horsley Park hazardous materials remain below that maximum allowable storage as shown in below Table. The raw materials on site are converted into non-hazardous material due to volume of water content. A chemical register is kept and maintained for all hazardous and dangerous good materials with location outlined on site manifest (Appendices 1) . Safety Data Sheets (SDS’s) for each dangerous goods and site manifest are stored in HAZCHEM emergency box On the front gate eastern side of building (Gate C, Coordinates (-33.8339977,150.8232814)). The Site Manifest is updated by the site annually or when required by legislation. The dangerous goods are stored in raw material liquid shed (Warehouse H) that was designed as containment/ bunded area. Class 3 materials are stored in flammable storage containers located on hardstand protected inside unloading station. In the event of a chemical spill in unloading stations, these are cleaned immediately and contained within bunded area. The drains are automatically shut off and controlled by an actuator that sensors tanker arriving in each bay and verified by employee prior to discharge into storage tanks. Storage tanks are located inside the facility in a containment bund. If a spill occurs, depending on the type of dangerous goods and waste-water treatment capability, chemicals will be recovered into containers then removed offsite by Road Tanker to an appropriately licenced waste disposal facility. Below is the list of maximum storage quantities of dangerous goods permitted for site table 1 and hazardous chemicals stored in designated location table 2. Table 1: Maximum Storage Quantity of Dangerous Goods

Class; PG; Description; Quantity (L); Storage Area. 2.1; N/A; Flammable gases – LPG; 3,920; Bulk Tank. 3; II/III; Flammable liquids; 32,000; Class 3 DG Storage Area. 4.1; II/III; Flammable solid. C1; N/A; Combustible liquid. C2; N/A. 5.1; II; Combustible liquid; 44,000; Liquid Storage Shed. 1,000; DAF. 6.1; II; Toxic substances; 5,000; Liquid Storage Shed. 8; II/III; Corrosive substances – acids and bases; 160,000; Tank Farm. 8; II/III; Corrosive substance; 55,000; Liquid Storage Shed. 8; II; Corrosive substances; 1,000; Liquid Storage Shed. 9; III; Environmentally hazardous substances; 150,000; Liquid Storage Shed. 9; III; Miscellaneous DG’s; 30,000; Tank Farm.

Table 2: Hazardous Chemical Storage HAZARDOUS CHEMICALS STORED IN PRODUCTION – DG Cabinet HSFL. STORAGE#; TYPE; MAXIMUM STORAGE CAPACITY (L); UN #; SHIPPING NAME; CLASS; PG; PRODUCT NAME; HAZCHEM CODE; TYPICAL QUANTITY (L). DG Cabinet - HSFL; Liquid; 5; 1210; PRINTING INK, flammable or PRINTING INK RELATED MATERIAL (including printing ink thinning or reducing compound), flammable nitric acid, hydrofluoric acid); 3; II; Linx Black Fast-Drying Ink 1240; •3YE; 2. DG Cabinet - HSFL; Liquid; 21; 1210; PRINTING INK, flammable or PRINTING INK RELATED MATERIAL (including printing ink thinning or reducing compound), flammable; 3; II; Linx Fast-Drying Solvent 1512; •3YE; 20. HAZARDOUS CHEMICALS STORED TRADE WASTE TREATMENT – DAF PLANT. STORAGE#; TYPE; MAXIMUM STORAGE CAPACITY (L); UN #; SHIPPING NAME; CLASS; PG; PRODUCT NAME; HAZCHEM CODE; TYPICAL QUANTITY (L). SB1; IBC; 1000; 1824; SODIUM HYDROXIDE SOLUTION; 8; II; CS50 (SODIUM HYDROXIDE 50%); 2R; 1000. SB2; IBC; 1000; 2014; HYDROGEN PEROXIDE, AQUEOUS SOLUTION 20% - 60% HYDROGEN PEROXIDE; 5.1 SUB 8; II; CWT BT 202 (SODIUM PEROXIDE 50% SOLUTION); 2P; 1000. SB3; IBC; 1000; 3264; CORROSIVE LIQUID, ACIDIC, INORGANIC, N.O.S. (Contains Ferrous sulphate); 8; III; CLEARFLOX 91207 (FERROUS SUPLHATE 7%); 2X; 1000. SB4; IBC; 1000; 2693; BISULPHITES, AQUEOUS SOLUTION, N.O.S.; 8; III; RO 100 (Sodium Metabisulfite Solution); 2X; 1000. SB5; IBC; 1000; 1830; SULPHURIC ACID with more than 51% acid; 8; II; SA 70 (SULPHURIC ACID with more than 51% acid); 2P; 1000. ST = Storage Tank, BT = Balance Tank, PT = Polymer Tank, SB = Spill Bund, N/A = Not Applicable.

HAZARDOUS CHEMICALS STORED IN TANK FARM – ABOVE GROUND TANK. STORAGE#; TYPE; MAXIMUM STORAGE CAPACITY (L); UN #; SHIPPING NAME; CLASS; PG; PRODUCT NAME; HAZCHEM CODE; TYPICAL QUANTITY (L). TK-14; AGT; 22,000; 1791; HYPOCHLORITE SOLUTION; 8; III; SODIUM HYPOCHLORITE; 2X; 19,800. TK-13; AGT; 22,000; 1791; HYPOCHLORITE SOLUTION; 8; III; SODIUM HYPOCHLORITE; 2X; 19,800. TK-12; AGT; 30,000; 1791; HYPOCHLORITE SOLUTION; 8; III; SODIUM HYPOCHLORITE; 2X; 27,000. TK-11; AGT; 30,000; 1824; SODIUM HYDROXIDE SOLUTION; 8; II; SODIUM HYDROXIDE 50% W/W; 2R; 27,000. TK-02; AGT; 50,000; 2586; ALKYLSULPHONIC ACIDS, LIQUID OR ARYLSULPHONIC ACIDS; 8; III; GARDILENE SSAS; 2X; 45,000. TK-01; AGT; 30,000; 1760; CORROSIVE LIQUID, N.O.S.; 8; II; GARDIQUAT 1450; 2X; 27,000. ST = Storage Tank, BT = Buffer Tank, AGT = Above Ground Tank; UST= Underground Storage Tank; N/A = Not Applicable.

Safety Equipment

The safety critical equipment implemented at Jalco Australia Pty Ltd Horsley Park site include emergency stops to automatically shut off valves for manufacturing or filling units, alarm system, high level sensors. Emergency stop or shutdown applications are located at multiple locations across all production filling lines, manufacturing vessels, piping systems, pressure vessels and boiler unit. Spillage are contained in bunded area and captured in pit/s that is transferred to DAF plant for treatment. In a worst case scenario event whereby an uncontrollable release of material exists the site, the 2350 tonne underground OSD Tank shall be closed at pit 111/1 to prevent contaminants polluting stormwater drainage. The site will arrange for a certified waste provider to pump material from OSD tank. Pit 111/1 is located at UPS Healthcare entrance at Unit 2/8 Johnston Crescent. In addition, there are interlock, monitoring sensors, alarm systems applied to plant and equipment to alert employees of emergencies that surface. The pumps and valves on site are designed with no reverse flow that avoids release of material that cause harm to people and environment. Instruments installed are available on tanks to control chemical overflows that is linked to alarm when there’s an issue with processes. The safety critical equipment are captured in a risk assessment and embedded preventative maintenance program to enable routine checks for functionality and efficacy. Personal protective equipment (PPE) is available in accordance with SDS especially for corrosive chemicals. PPE located at unloading stations, DAF and manufacturing areas include safety googles EN166 or ASNZ 1337.1, chemical grade full length gloves EN374, face shield, PVC apron and protective garment PVC EN14605 or EN13982. Safety Data Sheets are available for all raw chemicals and finished goods products and are captured on chemical register for site. Chemical risk assessment is also completed for all raw chemicals and finished goods products. Dangerous goods are labelled with UN information, pictogram and emergency contacts ( this also includes chemical storage tanks for non-dangerous good materials). Emergency Control team including first aiders will wear the following caps during an emergency: Emergency Control Team; Cap Colour. Chief or Deputy Warden; White. Warden; Red. First Aider; Green. In an emergency a first aid kit and defibrillator will be taken to emergency assembly point. Fire Fighting Equipment on premises include; Fire hose reels, Fire extinguisher, Hydrants, Exit lights, Exit Signages, Fire Indicator Panel and Sprinklers. Evacuation diagrams are available at each fire exit door to assist employees, visitors and contractor in an emergency. Eye wash stations and showers are installed at unloading stations, production filling lines, manufacturing mixing tanks, enzyme and dispensary areas. The eye wash stations are positioned 10 seconds from hazard as per AS4775. 240L Hazchem Spill kits (booms, absorbent pads, chemical compatibility absorbent material, PPE, anti-static shovels) are available across the premise including drain covers for hardstand area. General Manager of Manufacturing Operations or nominated delegate is responsible for notifying authorised listed above.

Communicating with Neighbours and the Local Community

Refer to above section “Notification of Neighbours and Local Community”. The site will organise community engagement to meet and greet commercial business on Johnston Crescent and residents based on greenway place, Horsley Park. Information of site processes is available on Jalco website for commercial and neighbours unable to attend site orientation.

Minimising Harm to Persons on the Premises

Maps. Refer to appendices section for maps.. In case of an incident, all staff, visitors and contractors must follow the instructions of pollution incident control coordinators and emergency control. Unless instructed otherwise, everyone on site ( including pollution incident control coordinators and emergency control) must immediately evacuate to emergency assembly areas located outside Gate A (main building and production) and Gate C (warehouse and collation). Using whatever means they have at their disposal, pollution incident control coordinators and emergency control shall ascertain numbers of staff, visitors and contractors present on site. Wardens will conduct a sweep of the site and once an office ( including amenities), manufacturing, dispensary, production filling and warehouse is checked and cleared, it must be tagged out to ensure the facility is fully evacuated and all people on site are accounted for. No-one shall re-enter the site until emergency services or site Chief or Deputy Wardens give the all clear signal.

Actions to be Taken During or Immediately After a Pollution Incident

IMMEDIATE CONSIDERATION - If safe to do so contain the spill, leak or escape of pollutant - In the event of a pollution incident, the spill leak or escape of pollutants will be controlled and contained. If operation of onsite plant or pumps is causing the pollution incident, then the affected system will be shutdown. - Call 000 in an Emergency - The site must contact 000 if the incident presents an immediate threat to human health or property. Fire and Rescue NSW, the NSW Police and the NSW Ambulance Service are the first responders, as they are responsible for controlling and containing incidents. INTERNAL NOTIFICATION - Report to Supervisor - Prior to any other action, the initial observer will report the issue immediately to their supervisor or the Operations Manager. - Operations Manager must follow PACT Group reporting protocols and protocols highlighted in JAHP-ERP-001 Emergency Response Plan NOTIFICATION TO THE REGULATOR - Determine if a Notifiable Incident - After the initial response to any events that may cause immediate harm to human health or property the Workplace Manager will determine if the event constitutes an “actual or potential material harm incident” in consultation with relevant stake holders. - Notify Authorities - In the event of a “material harm incident” the following authorities will be contacted as per notification of relevant authorities immediately after becoming aware of the incident, under the legislative duty to notify: - EPA - Fairfield City Council - NSW Ministry of Health - SafeWork NSW - NSW Fire and Rescue - Sydney Water St Mary In the case of a “material harm incident” the following information will be noted and forwarded to the authorities when they are notified of the incident: - Time and date. - Nature and location of the incident. - Duration of the incident. - Location of areas that may be affected by the pollution incident. - Pollutant involved and the estimated quantity/volume and concentration - Circumstances in which the incident occurred. - The proposed action to be taken in dealing with the pollutant and any further incidents that may result. A detailed record will be kept of all steps involved in dealing with each incident and kept on site in case additional information is required. After the initial notification of a material harm incident, it is the responsibility of the Pollution Incident and Control Coordinators to coordinate with any authority that is contacted. RESPONSE TO INCIDENT If the incident does not pose any threat to human health or property, all possible actions will be taken to control the pollution incident and minimize health, safety and environmental consequences. These actions will be employed to the maximum extent possible to: - Provide for the safety of people at and within the vicinity of the site; and - Clean-up the pollution incident.

The Jalco Australia Horsley Park emergency evacuation assembly point is located on footpath in front of main building [Gate A] and warehouse [Gate C]. There is an emergency assembly point signage at both locations. In the event of an evacuation the Jalco Australia Pty Ltd Horsley Park Emergency Response Plan needs to be followed (JAHP-ERP-001 Emergency Response Plan). DUTIES DURING AN EMERGENCY EVACUATION Managers that have not been designated for a particular role (e.g., Chief Warden) are responsible for the following duties during an evacuation: Managers are responsible for helping Area Wardens evacuate their areas. Managers shall provide support to ensure staff immediately evacuate and proceed directly to the evacuation assembly area. Managers shall report to the Area Warden and have their names recorded. They are then to assist in the confirmation that all staff have been evacuated and to ensure that the staff remain at the assembly point until approved to leave. Managers shall then be “on call” to help any duties required by the “Chief Warden”. Managers shall assist in diverting away deliveries or visitors to the site during the emergency. The backup facility for Jalco Australia Pty Ltd Horsley Park, is Jalco Promotional Packaging located at 10 Interchange Drive, Eastern Creek. Where possible, deliveries should be diverted to this site after consultation with the receiving department and Distribution Centre Manager at Eastern Creek. Managers are to consult with the General Manager NSW Operations about the need for delaying or cancelling any following shift, if the emergency is thought to close the plant for an extended delay. DUTIES AFTER THE EMERGENCY Management shall collaborate with Chief Warden when emergency has terminated and satisfied health, safety and environmental requirements. This includes: The emergency services return control to site Chief Warden or Emergency Plan Committee Declaration by Chief Warden in liaison with Emergency Plan Committee that emergency has been terminated. Management is responsible for assessing the impact of the emergency on company property, stock, work in progress, and delays in orders. This must be quantified as part of any insurance claims. Management is responsible for taking action to get the plant safe and operational again. RELEASE OF INFORMATION In case of a Major Emergency only a Company Director or Contract Manufacturing Executive General Manager may release information to the public, e.g., media, customers, suppliers etc.

A description of actions taken to combat pollution caused by the incident are described in each relevant listed incident safety section.

Coordinating with Persons

The Chief Warden shall ensure that the required emergency services and Company General Manager have been called and the evacuation siren activated. The Chief Warden will assume overall supervision of the site during an evacuation of personnel and liaise with the attending Emergency Service (e.g., Fire Brigade, Ambulance), upon their arrival. The Chief Warden must liaise with Area Wardens to ensure all personnel are accounted for. Obtain the site manifest (if applicable) and have ready to present to Emergency Services on arrival. Liaise with the site Maintenance Manager concerning site services actions. E.g., isolation of gas, electricity, closure of storm water gates Liaise with First Aiders to determine if there are any injured personnel. Direct additional Evacuation Procedures to those initiated by Area Wardens. The Chief Warden is to liaise with Emergency Services and update them with: - Location of all personnel and details of anyone not accounted for; - What the emergency situation is and where; - Location of Dangerous Goods (Provide manifest if applicable to site for dangerous goods); - Status of site services (e.g., gas mains shut, electricity); and - Advise of any injured personnel - Provide assistance to Emergency Services for the duration of the emergency; and - Maintain regular contact with Company General Manager Manufacturing Operations as to current situation. The Chief Warden, with the help of Area Wardens and Company Management, shall ensure that no personnel leave the assembly point to retrieve personnel items or vehicles during the emergency unless authorized by Emergency Services. The Chief Warden shall wait for clearance from Emergency Services before informing personnel when they may return to specific areas or all areas, inform the Wardens concerned to issue appropriate instructions.

The Chief Warden will assume overall supervision of the site during an evacuation of personnel and liaise with the attending Emergency Service (e.g., Fire Brigade, Ambulance), upon their arrival. The Chief Warden will provide assistance to emergency services. General Manager NSW Manufacturing Operations is the emergency commander liaising with Executive General Manager, Pact Group risk team and legal team. General Manager NSW Manufacturing Operations is responsible to activate emergency plan for major catastrophes, notify regulatory bodies, customers, suppliers, media and manage process unless otherwise delegated to alternate General Manager or site personnel.

| Staff Training |  |  |  |  |
| --- | --- | --- | --- | --- |
| Staff will undertake desktop exercise or scenario, as well as participate in drills and practical exercises. The objective of staff training is as follows: All Employees (including contractors) – - Understand pollution incident procedures, their roles, responsibilities and how to activate these in a pollution incident situation. - Contents provided within pollution incident response plan in the form of a site presentation. Cross Functional Team (Department Leads) – - Response teams have detailed understanding of their roles, how to support each other, mobilise, work together to resolve the pollution incident. The training will include formal training and desktop/ practical scenarios. - Contents provided within pollution incident response plan. Note: Individuals heavily involved in dealing with chemicals are required to undertake formal practical training and/or drills. Records of staff training will be maintained on site. |  |  |  |  |
| Testing and updating of the PIRMP |  |  |  |  |
| The testing of this pollution incident response plan will be undertaken on an annual basis during the life of EPA License. Testing will be undertaken in the form of desktop simulations, practical exercises and drills at Jalco Australia Pty Ltd Horsley Park site. The pollution incident response plan will be tested within one month of any pollution incident occurring or at least once every 12 month period. In the circumstance significant changes occur such as increase in production capacity, new plant and equipment, significant upgrades and installation plan will be reviewed. In addition, a new risk assessment will need to be completed to reflect changes. Pollution incident response stimulation and practical exercise records will be maintained at Horsley Park site and revisions available in Document Control section of this plan. Pollution Incident Response Plan will be reviewed following each mock incident to ensure it remains up to date with current site conditions. Furthermore, Fire and Rescue NSW will be invited to participate in or observe pollution incident response plan in the first year of EPL issuance and significant changes to plant and equipment to site. This will allow Fire and Rescue NSW to raise their awareness of the site, provide feedback and update their incident plans. |  |  |  |  |
| PIRMP Testing Details |  |  |  |  |
| Date tested | Tested by (to include the names of all people involved in testing) | Details of test (e.g., nature of the test, involvement of other agencies) Note: Testing must cover all components of the plan. | Finding of test, including issues identified | Next scheduled testing date (must be within 12 months from current test) |
| Document Control Register. Date; Reason for update (e.g., address issues identified in testing, contact details/personnel have changed); Details of updates (nature of changes to PIRMP); Date the updated version uploaded to website (if applicable); Author. 20/11/2023; New; New document established for Jalco Australia Horsley Park site. In consultation with Gopi Dhanekula (Engineering Manager and site Chief Warden), Aaron Colautti ( Pact Group Environmental Manager), Tina Nicolitsis ( Regional Health, Safety and Environmental Manager), Rojli Rajon ( Site Operations Manager).; November 2023; Christine Kazzi. Appendices. Appendices 1: Pollution Incident Response Management Mock Incident Form Note: This is only a Mock Pollution Incident Exercise to test PIRMP. PIRMP may be updated based on the findings of the test.. Date of Exercise:. Start Time:; Finish Time:. Mock Pollution Incident Coordination Team Members. Leader. Member. Member. Member. Member. Member. Member. Member. Description of Mock Pollution Incident Scenario. Pollutant Name; Type of Pollution (Air/ Noise/ Water/ Land etc.); Approximate amount of pollutant release. Potential Risks of Pollution. Immediate actions taken. External Notification: □ Yes □ No (Check Internal Notification Guideline). Notification to be done by:. □ EPA □ Council □ SafeWork NSW □ Ministry of Health □ Fire & Rescue NSW. Notification to Neighbour: □ Face to face contact or telephone call □ Letterbox drops □ Publication of updates on Jalco’s Website □ Emailing of updates □ Doorknocking. PIRMP Section Tested and Findings. Section; Finding. Notification of incident. Inventory of Pollutants. Safety Equipment on Site. Control Measures to minimise the risk of pollution. Actions taken during and immediately after an incident. Staff Training. General Comments (including improvement required on response, PIRMP document update, debrief questions: what worked?, what would we do the same next time? What would we do differently next time? What needs did we identify? Staff training, safety procedures, additional equipment etc.). |

Appendices 2: Site Manifest

Note:  Double click on image to view in A4 legible font

Appendices 3: Dangerous Goods Plan

Note: Double click on image to view in A4 legible font

Appendices 4: Stormwater Plan

Note: Double click on image to view in A4 legible font

Appendices 5: Jalco Australia Pty Ltd – Neighbours

Note: Double click on image to view in A4 legible font